ADS-B & ADS-C Trajectory Fusion — Gap-Filling and Route Analytics

Noel Abd Al Karim (241002) · Mark Harb (210003) · Matheos Touma (231250) · Fayez Sammour (233540) — ESIB-USJ, AeroML3, Spring 2026

| Data Source ADS-B data was obtained from the OpenSky Network — a community sensor network providing dense position updates over land. ADS-C (satellite-based) was also sourced from OpenSky and covers oceanic gaps where no ground receivers exist. Each flight is stored as three parquet files: ADS-B before, ADS-C waypoints (held-out ground truth), and ADS-B after. Stat; Value. Total flights; 2,149. Gap duration; 170–516 min (mean 267). ADS-C waypoints; 3–30 per gap. Coverage; N. Atlantic, Pacific, Indian Ocean. |  | Approach Rather than directly reconstructing trajectories, the BiGRU learns to correct great-circle interpolation predictions based on real flight data. It predicts the residual — how much the great-circle guess needs to be adjusted to match the actual trajectory. Reconstruction is performed from both sides of the gap: the model predicts forward from the ADS-B before the gap and backward from the ADS-B after, then blends both predictions linearly. |
| --- | --- | --- |

Methods

| Baseline — Great-Circle Interpolates N evenly-spaced points along the shortest geodesic arc between the last known ADS-B position before the gap and the first known position after. Assumes constant speed along the arc. Simplest reference method requiring no training data. |  | Kalman Filter (RTS Smoother) Operates in along-track / cross-track coordinates. Uses 64 resampled ADS-B points before and 32 after as Kalman observations. Full forward pass followed by RTS backward smoother. Hyperparameters tuned on validation split via grid search (measurement_std=250m, accel_along=0.005, accel_cross=0.001 m/s²). |  | BiGRU — Sequence Model Input: sequence of 10 points × 7 features (lat, lon, alt, vx, vy, accel, turn_rate) — lat,long,alt relative to the last known point. Bidirectional GRU (hidden=128, layers=2) + attention + dropout (p=0.3) + FC head (256→128→ReLU→3). Predicts residual correction on top of great-circle extrapolation. |
| --- | --- | --- | --- | --- |

| Evaluation Results Evaluated on held-out ADS-C waypoints — never seen during reconstruction. Geodesic error = mean closest-point distance to ADS-C waypoints. Method; Baseline (great-circle); Kalman RTS smoother; BiGRU model. Mean (km); 140.27; 111.27; 95.30. Median; 109.79; 88.23; 70.50. Improved; 19.6%; -20.7%; 32.1%. * BiGRU residual errors: lat 0.0009–0.5324°, lon 0.0051–1.0086°. Full geodesic km evaluation pending. The Kalman RTS smoother reduces error by 20.7% (mean) and 19.6% (median). Improvement peaks at the gap midpoint (τ=0.5) where the bidirectional context is most beneficial. Best single-flight: 154.21→73.61 km (52.3%). |  | Why Better Tracks Matter Raw ADS-B misses the entire oceanic gap, systematically underestimating route distance and CO₂ emissions. Method; Route (km); CO₂ (kg). Raw ADS-B (no gap); ~5,800; ~110,200. Reconstructed; ~8,900; ~169,100. ADS-C truth; ~9,100; ~172,900. CO₂ proxy: 19 kg/km (ICAO widebody average). A fleet of 100 daily long-haul flights with untracked oceanic segments accumulates ~2.15 billion kg CO₂/year unaccounted. System & Monitoring Packaged as reconstruction.py — run per flight from the command line. Outputs baseline, Kalman and BiGRU parquet files + GeoJSON for geojson.io visualization. Monitoring tracks gap rate, reconstruction quality, and per-flight error drift across the full dataset. |
| --- | --- | --- |

References: OpenSky Network API — opensky-network.org · Olive et al., ADS-C Fusion, arXiv:2505.06254 · Sun et al., ADS-B Preprocessing, JOAS 2023 · Welch & Bishop, Kalman Filter, UNC 2006